## FOOD LION REMODEL - REMODEL BID WALK THROUGH NOTES

## STORE #1662      Laurel, MD

DATE OF SCHEDULE PROVIDED AT BID:  3/17/2020

STORE HOURS = 7am – 10pm

_________________________________________________________________________________

Important note:

ALL attached bid notes are specific and will override the drawings. General Contractor is responsible for the timely distribution of updated information to ALL sub’s and field personnel.

_________________________________________________________________________________

Standard Notes:

- The bid date for this project is 3/31/2020 at 2:00 pm.  This project will be bid through Retail Business Service's Project Management Site, PMWeb  https://rbs.pmweb.com.  For questions or support contact: Shana Hines at 704-310-4218 or shana.hines@retailbusinessservices.com

- Landlord Information: (Leased)

Uriel Ocha-Diaz

Albaluz, LLC

1013 Parrs Ridge Drive

Spencerville, MD  20868

Albertoochoa24@yahoo.com

202-438-1568

- All bids must contain name of plumbing, electrical, and HVAC contractor and their bid price.  Please 
submit all qualified sub prices and include a minimum of at least three per trade.  A copy of the bid 
cover page, showing the bid amount, is required.

- Food Lion will provide Builders Risk and Owners Protective Liability Insurance.  The contractor must provide liability insurance.

- PLEASE NOTE!  Contractor MUST include a payment and performance bond in the bid price IF THE JOB IS PRICED OVER $500,000.  The bonding company’s A M Best Rating must be B+6 to A-6.  Proof of this rating must accompany bond.  Bond rating MUST be within the past 30 days.

- GENERAL CONTRACTOR TO INCLUDE PERMIT FEES IN BID.

- Retail Business Services reserves the right to accept any bid or bid alternate, to reject any or all bids, or to waive any informalities in bids received where such acceptance, rejections, or waiver is considered to be in the best interest of Retail Business Services.

- IMPORTANT NOTE:  The awarded General Contractor must submit a “Schedule of Values” for this project when he submits his contract to the Retail Business Services Construction Project Coordinator.

- General Contractor shall provide unit pricing in bid proposal for the following items:

  - Rock excavation (Cubic yard).

  - Finished concrete (cubic yard).

  - Bad soil removal and replacement (cubic yards).

  - Asphalt removal and replacement (square yard).

- Food Lion’s standard new store specifications will govern all workmanship and materials. The General Contractor will be required to provide the bidding sub-contractors a copy of the store specifications which will be adhered to during construction. Any questions or discrepancies must be confirmed through the Construction Manager.

- IMPORTANT NOTE:  The General Contractor must incorporate the new Work Flow billing procedures as set up by Retail Business Services Construction. Please contact Lori Bowen (704-310-3036) for any questions or concerns related to billing.

- IMPORTANT NOTE:  The awarded General Contractor is required to submit a partial Release of Lien, equal to the amount of the pay request, to accompany each Contractor’s Application for Payment.  Payment will not be made until each pay request has this information attached.

- General Contractor shall submit each change order within 45 days of the change order work completing.  Estimated change order cost should be communicated to the Construction Manager at the time the change order is communicated.

- General Contractor shall submit to Owner an application for Final Payment; all Change Orders are to be submitted and processed before submittal of the application for Final Payment. In the event General Contractor shall fail to make and present such application within one hundred eighty (180) days of Grand opening of a remodel or takeover, then any and all applications for such Final Payment shall be, and the same hereby are, waived and forever barred from collection by General Contractor from Owner.

- General Contractor shall video tape or photograph existing exterior conditions at store prior to any work beginning.  This should especially include the conditions of the roof, asphalt drives, landscaped and grass areas, and parking areas.  Any backcharges by the landlord that cannot be proven false by existing photographs will be paid by the General Contractor.

- General Contractor to provide on-site Superintendents with a digital camera for photos of all night work performed regarding trench lines, hub drains, etc.

- General Contractor to verify with local authorities and obtain/purchase any noise ordinance permits required.

- Drawings for Retail Business Services projects are housed on the company’s Project Management Site, PM Web. Each General Contractor is responsible for downloading their drawings (along with Refrigeration Summary, Asbestos Survey, Roof Survey and Food Lion Specifications) via the website. https://rbs.pmweb.com . There is no fee to download drawings from the site. For questions or support contact: Shana Hines at 704-310-4218 or shana.hines@retailbusinessservices.com

- General Contractor is responsible for verifying, obtaining, and incorporating the most current fixture plan and/or drawing revisions and distributing to the subs. Failure to obtain the most current fixture plan will result in the General Contractor having to make the necessary changes at their cost.

- General Contractor must insure that all subs related to the remodel; receive a copy of the refrigeration summary, asbestos survey, Food Lion Specification Book, etc. so the sub can research and incorporate a price appropriately for the work to include labor and material.

- General Contractor is responsible for providing a Superintendent for all subcontracted work under their contract.  A qualified Assistant Superintendent should be used for supervision above reasonable working hours of the Superintendent.  General Contractor is to be on site for the Construction Manager's weekly management site visit, and for all daytime inspections.

- General Contractor shall not remove the project Superintendent at any time unless approved by the appropriate RBS Construction Manager through completion of the project.

- General Contractor will be totally responsible for all construction and equipment related material that comes to the job site. Individuals who sign for equipment must review the material for any damages and must also insure that everything is accounted for in the shipment. If this is not done with accurate detail; the General Contractor will be billed for equipment replacements. If there are concerns with the shipment then contact the appropriate Construction Manager.  Refrigeration Contractor to be responsible for refrigerated items.

- General Contractor’s Project Manager is required to meet with the Construction Manager on a bi-weekly basis on the job site.

- A weekly meeting is required between the General Contractor and Store Manager to discuss the next (2) weeks work.  During scheduled night work, a meeting is required between the Safety PIC person and the GC to discuss safety items during that night's work.

- General Contractor must have all punch list items complete within three weeks of receiving it. If not, Retail Business Services Construction reserves the right to hire someone to complete the work at the General Contractors cost.

- Retail Business Services Construction reserves the right to back charge subcontractors which do not show up for scheduled night work.  This may include labor cost for scheduled store personnel, store operating cost, etc.

- General Contractor to provide the store with a 24 hr. Emergency Contact List of all subs at the pre-construction meeting. General Contractor/Sub-Contractors to assume maintenance responsibility for the store at this point.  Emergency calls will be handled as such.  Any additional charges outside of the scope of work will be billed as a Maintenance Ticket.  A Maintenance Ticket must be entered to get paid.

- Smoking, eating, and drinking will only be allowed in an area designated by the Retail Business Services Construction Manager and the Store Manager.  General Contractor is to provide contracting subs a butt bucket for the rear exterior. All contractors are required to maintain their in-store purchase receipts and taped to the product while in the store.

- Shopping carts/floats are not to be used for construction purpose unless authorized by the Construction Manager.  Construction Carts may be provided, coordinate with Construction Manager.

- General Contractor will be responsible for “ALL” dumpster dumps including those for the Refrigeration Contractor and Grocery Reset.  A Food Lion Vendor will be responsible for removal of shelving metals, General Contractor will be responsible for all other discarded material during Grocery Reset.  Grocery reset may be completed prior to start of construction.  If this occurs, Retail Business Services provides dumpsters.

- General Contractor must use professional signage and neatly installed orange fencing for any detours required during the course of the remodel. Coordinate the use of dust walls vs. dust screens with Construction Manager.  Use 2x4 framed plywood dust walls with Tyvek (lettering turned inward and upside down) when any jack hammering, sawing, demolitions, etc. is taking place.  Dust walls need to be structural, lockable, and insulated where appropriate.  Dust screens should be constructed of 6mil. poly or painter’s cloth attached to the ceiling and secured at the floor by rolling a 2x4 in the excess material.  Dust screens are to be used for light construction to control dust and visibility only.  Food Lion grocery carts or wet floor signs are not allowed to be used as fencing supports.

- Retail Business Services will be supplying the Automatic Doors, Control Panels, Interior Lighting, Impact Doors, Sinks, Water Fountains, Water Heaters, Aqua stat & Stands, Hand Dryers and HVAC equipment.  Installation and warranties of this equipment is to be supplied by the General Contractor.  The General Contractor is responsible for ensuring that the SUB-CONTRACTORS order these items directly from the supplier as soon as the contract is awarded.  These items must be stored on site in the Sub-Contractors storage trailers.  A complete package with all the information will be sent to the awarded General Contractor.  National Account items MUST BE EXCLUDED from your bid, with the exception of installation and warranty.  General Contractor’s Project Manager must confirm that ALL orders have been made with delivery dates and provide documentation at the Pre-Construction Meeting.  First Pay Draw will not be released until all material orders have been verified. 

Please ENSURE you are copying the Equipment Coordinator noted on the order you place to the vendor as noted below for BETTER tracking.

Automatic Doors – Assa-Abloy / Besam – Jessica Shane @ 1-609-249-9498
Email:  Jessica.Shane@assaabloy.com; David.Hastings@retailbusinessservices.com

Power Wall-Electrical Switchgear – CD Controls – Contact: Ed Rios @ 706-476-7303 (Cell), Email:  edrios@cdcontrols.com

HVAC System/Energy Management/ Environmental Control – Sesco/McNamara – Contact: Erin Mariano @ 336-996-2220, Email erin.mariano@trs-sesco.com or Penny Shepherd @ 336-996-2220, Email: penny@trs-sesco.com

Interior Lighting Package – Sylvania Lighting – Contact: Lisa Mastin @ 1-800-579-1514

Email: lisab.mastin@osram.com

Site Lighting Package – Lighting Service Design/LSD – Contact:  Rick Donald @ 804-357-7840; Email:  rickdonald@lightingsolutions-design.com

Hollow Metal Doors & Hardware – B.A. Hoft – Contact: Joseph Holland @ 919-596-4446
Email:  jholland@bahoft.com

Dock Doors, Seals & Dock Leveler – Overhead Door – Contact: Amanda Cumberland 
@ 469-549-7025, Email:   Amanda_Cumberland@OverheadDoor.com; David.Hastings@retailbusinessservices.com

Traffic Doors – Hefco – Contact: Lisa Windle  @ 800-327-1817

Email:  cs4@hefcollc.com; David.Hastings@retailbusinessservices.com

Vestibule Carpet – Portico – Contact:  Bill Long @ 877-285-8899 x305

Email:  blong@porticosystems.com

Hand Dryers – George Scott & Associates – Contact: Bill Scott @ 704-376-4775

Email:  bill.scott@gsaclt.com

Sinks – John Boos – Contact: Lisa Windle @ 800-327-1817

Email:  cs4@hefcollc.com

Water Fountains, Water Heaters, Stands, and Aquastat (Therma-Stor) – WW Grainger  
Contact:  David.Hastings@retailbusinessservices.com; natalie.lebron@grainer.com

NOTE:  General Contractor to provide a list in each bid of the National Account items Retail Business Services is to supply.

- Plumbing Contractor is to order all new equipment through the National Account program, ALL other required repair parts can be purchased by the plumber outside of the National Accounts program once verified through the Construction Manager.

- Carpet mats are to be clean, same in color, and have rubber edges.  All mats are to be taped in place at all times. Mats are to be supplied by the General Contractor, do not use store's carpets.

- Any ceiling cleaning must be done by a professional cleaning company.  Ceiling cleaning contractors shall use cleaning mixtures that expose the ceiling tile to no more than 10 grams of water solution per square foot of ceiling tile, shall ensure that cleaning process does not create sagging or damage to the ceiling tile, and shall ensure that the operation will not adversely affect the performance or dimensional stability of the ceiling tile.  It will be the ceiling cleaning contractor’s responsibility to cover and protect the store product during the cleaning process.  The ceiling cleaning contractor shall not use any cleaning solutions that would have a corrosive effect on the ceiling grid or other metal surfaces in the store, which would eventually lead to the rusting or oxidizing of the metal surfaces.  The Ceiling Cleaning Contractor shall send a Material Safety Data Sheet (MSDS) for each store to Retail Business Services for our files as well as leaving a MSDS at each store while work is being done.

Approved ceiling contractors:

Keep Looking Up Acoustical Ceiling Cleaning & Services

Contact: Michael Tuczynski

Ph: Michael - 910-793-8200 or 910-443-6864

E-mail: cleaningk@bellsouth.net or mjt_klu@bellsouth.net

Team ATG

Contact:  Angie Kirsch

Ph: 800-334-1789 or 740-317-4204

Email:  angie@teamatg.com

Southern Cleaning

Contact:  Dennis Wilczewski

Ph: 864-325-1094

Email:  southerncleaning@rocketmail.com

Carolina Ceiling Cleaning, Inc.

Contact:  Mike Allred

Ph:  336-953-5433

Email:  mike@carolinaceilingclean.com

- Electrical Contractor to provide a Commercial Qualified Lead Electrician for each project.  This person is to be identified as the Project Electrician and is to work on site a minimum of 40 hours each week starting from week 1 of the Construction Schedule to 100% complete.  Exceptions to this are Reset week and Holidays if no work is scheduled, special circumstances can be approved by Construction Manager only.

- Electrical Contractor shall install all vertical low voltage cabling in finished and unfinished areas in EMT with a 90° bend at the bar joist in the direction of the cable run.

- Electrical Contractor shall install and adequately secure ALL covers.  Provide or replace covers and fasteners and plug any holes as required.  Device covers in Meat, Produce, Deli/Bakery, and Exterior shall be weatherproof (WP) and in proper condition at closeout.  Vacuum branch circuit panel board, SweatMiser, and LiteMiser interiors and clean the tops of all electrical panels, cabinets, and troughs.

- Electrical Contractor shall install all low voltage cabling running overhead in the bar joist and tied every 15 ft.  Cabling that is suspended from the bar joist or lying on a ceiling will not be accepted.  Electrical Contractor is to work with each Food Lion vendor to remove any and all abandoned low voltage cabling.

- A RBS Vendor Team will install the front-end equipment; this includes and is limited to:  all check stands, cigarette kiosk, and Customer Service counters.  This includes setting and handling all equipment, clean and dirty power, associated CAT5 and Beldon cables.  This work will be permitted separate from other work in this project by the RBS Vendor Team.

- Electrical Contractor is not allowed to use Armored Cable (BX or MC) for electrical runs.  EMT must be used to connect panel to junction box.  Armored Cable is only allowed as fixture whips or extensions from a junction box above sales area ceiling to cases/equipment below.

- Electrical Contractor is responsible for replacing and installing all missing power pole trim rings.

- Electrical Contractor to install Aluminum “In Use” receptacle covers when specified or required, plastic covers are not acceptable.

- General Contractor must use Contract Flooring (contact #800-356-6792 or #252-522-4588); Haywood Floor Covering, Inc. (contact #804-824-9741 or #804-815-2295), Tino Tile (contact #704-636-7735), for ALL store soft tile installation.  No other soft tile contractors will be accepted.

- General Contractor is to remove all old silencers and install new door silencers on all door frames after painting.

- General Contractor must install through bolts on new/relocated door closures.

- General Contractor must install commercial grade kick down door stops on all interior hollow metal doors.  (except front office inner door)

- General Contractor is to install kick plates on all doors that are missing or damaged.  Do not install on rear dsd/dock door.

- General Contractor shall caulk around all fixtures in restrooms/all corridors/produce prep/meat prep and deli/bakery area and all offices throughout the entire store – and/or accessories, which fasten to the wall.  (Examples:  water fountains, sinks, paper towel holders, electrical plate covers, plumbing fixtures, plumbing escutcheons, electrical outlets, sprinkler escutcheons and wall mounted fixtures, etc.)

- Painting Contractor is to follow the Food Lion National Account Paint program with Sherwin Williams Paint Company. Painting Contractor is to purchase all paint needed for this project (interior and exterior) from Sherwin Williams only.  Painting Contractor is to use the code “National Parent #C137” when ordering paint for this project.

- Painting Contractor is to properly paint Marlite as indicated in plans and bid notes by following the following procedures:  Prepare surface by cleaning Marlite and de-gloss surface using a 3M pad or ultra-fine grit sand paper.  Fill any holes, cracks, or imperfections with a wood filler or latex based caulk.  Apply prime coat "Shellac Primer BA9W8050 by using natural bristle or nylon/polyester brush, ¼" – ½" nap synthetic or mohair roller.  Next, lightly sand previously primed surface using fine grit sand paper.  Topcoat is to be 2 coats of "Zero VOC Acrylic B66-650 applied with nylon/polyester brush or 3/8" woven roller.  General Contractor is to verify each step is followed by providing digital photos of steps followed, failure to follow these steps along with pictures will result in new application of entire process at the General Contractor's expense.

- General Contractor is to use the following Marlite product when patching areas due to case/equipment moves prior to painting existing Marlite:  1001-S4 (paintable white with 4" grooves) or 1001-BG2 (paintable white with 2" grooves).

- General Contractor to use "Low Odor" acrylic flooring in areas defined on plans or bid notes.  "Protect 4300 PA" by Protective Industrial Polymers (contact – Steve McCamey 804-937-5000 activatedflooring@gmail.com) – OR – "Accelera HQ" by Dur-A-Flex (contact – Kevin Stephens 919-208-8662 kevins@dur-A-flex.com ).  Electrical/Plumbing/General Contractor shall bid all labor and material to disconnect and re-connect/adjust all equipment and fixtures to complete this flooring installation.

- In the best interest of our Customers, the following construction activities are limited to “night” work only and should be bid as such (night work is defined as 10 pm until 8 am):  Sales Area Painting, Sales Area Ceiling Tile Replacement/Cleaning, Sales Area Lighting Demo/Installation, Sales Area Framing and Drywall installation, Sales Area Drywall finishing, VCT Demo/Install, Sales Area Mechanical Duct Demo/Install, Sales Area Sprinkler Pipe Demo/Install, Entire Store Saw Cutting or Jack Hammering, Deli Floor and Wall Tile Demo, Restroom Floor and Wall Tile Demo, Restroom Plumbing work, Sales Area Masonry/Framing Grinding/Sawing.  All other dust or noise creating work on the sales floor shall be completed at night.  Any excavation in front of the store for Grease Traps, etc. is to be completed at night.  Coordinate with Construction Manager for approval of any work that would violate this requirement.  Store Manager has authority to halt work that he/she deems unsafe or disruptive to customers.

- Any new cable installed will be a Belden 1212 CMR Cat5e 4 pair 24 AWG (Belden 1200 DataTwist Cable Series).  New cables installed by the remodel Electrician are to be Blue in color and labelled using Duct Tape or paper tags.

- All 3rd Party Professional Cleaning as indicated in the bid notes for Deli final cleaning shall be completed by a Professional Cleaning Company approved by the Construction Manager for this project.  The following procedures are to be followed:

- The Cleaning Company will be required to use a commercial grade pressure/steam cleaner and shall follow Food Lions specs on emissions within the building. (No gas or propane equipment within the interior of the building)

- The Cleaning Company will be required to use a commercial grade kitchen degreaser. Safety Data sheets will be required to be available.

- Walls shall be scraped to remove any tape, caulk, and foreign residues.

- Walls shall be sprayed with a degreaser, rinsed, and then wiped down. Steam and pressure washing of the walls will be required as needed. A shield will be required for protection of the ceilings so as not to damage the ceiling tiles.

- Floors shall be sprayed down with a degreaser and use a surface cleaner to remove. Pressure wash/steamer will need to be used in areas of excessive dirt and buildup. Then use a squeegee to remove excess to the nearest drain and follow-up with mop pads to help dry the surface.

- Spray all the stationary equipment with a degreaser and wipe them down to a dry state. This shall include all pieces of equipment within the deli bakery.

- Final requirement is to have the PIC, Superintendent, and Construction Manager to sign off on the work performed and that it has been done in conjunction to the noted requirements above.

- Contractor to cover all equipment not being cleaned to protect from contamination or damage.

- Note: General Contractors are not allowed to self- perform this work without the written permission of the Construction Manager.

Turning Power Off to the Store

- If power is turned off to the store for any reason, the following steps are to be followed and the General Contractor is responsible for this process.

- Inform Retail Business Services Construction Manager (One week notice)

- Contact and inform Central Station (704 310 2537).

- Turn off all registers and computers individually.

- Turn off "CR" breaker.

- Turn UPS Switch to "Bypass".

- Turn off input breaker to UPS.

- When electrical work is complete and power is back up, turn breakers on.

- Contact Mike Peoples (336-613-3679) and he will walk you thru restarting the UPS unit as this involves some keystrokes, password, and switching the bypass switch.  If the UPS unit is rewired at all due to the "OW" change out the phasing must also be checked, which Mike can walk the Electrical Contractor thru as well.

- Turn on equipment.  A Manager needs to come in early to verify register function One Hour before the store opens.  Validate that all equipment (i.e. registers, workstations, etc.) are functional.  If any equipment fails to power up have the store contact Store Technical Services at 1-888-889-1105.

Safety Notes:

- General Contractor MUST maintain an OSHA approved safety program.

- Electrical Contractor will not be allowed to work on live electrical loads.

- General Contractor is to ensure that no emissions generating equipment is used inside the store (NO

GAS OR PROPANE EQUIPMENT)

- If it is required that emissions generating equipment must be used inside Food Lion, and if approved by

the Construction Manager, the following guidelines are to be followed as a minimum:

A letter of intent on the Contractor's Company Letterhead must be composed stating:

  - Type of equipment being used and its fuel type (Propane, Diesel, etc.)

  - Location (Store Address) approved for use including dates approved for use.

  - Name of Operating Contractor for equipment.

  - Personal Carbon Monoxide Measuring Device will be worn during operation of emission generating equipment.

  - Level of CO that will be maintained during operation.

  - Ventilation system description along with an illustrated drawing of air evacuation.

  - Verification that equipment has been inspected and certified within the last 6 months to operate within tolerable CO emissions.

----------------------------------------------------------------------------------------------------------------------------------------------

The following are department specific notes, please read thoroughly and coordinate with other store documents.

----------------------------------------------------------------------------------------------------------------------------------------------

Front Exterior

- General Contractor must install new signage on (3) existing cart corrals. General Contractor to remove the Damage Responsibility signs from the side of the existing cart corrals and grind off tabs.

- General Contractor is to replace (5) metal stakes and reattach handicap signs.

- General Contractor is responsible for hanging remodel banner supplied by Retail Business Services.  All exterior banners are to be mounted on the storefront parapet using a wood framed cradle.  The banner is to be framed front and back with 1x wood strips to sandwich the banner.  Banner/Cradle to be moves to different locations on the storefront to allow storefront work without removing the banner from customer view.

- General Contractor is to bid the removal and the disposal of the existing storefront signage from the EIFS front.  Remove all existing sign conduits and skim coat EIFS material to the nearest architectural element. No skim lines or divots will be accepted.  General Contractor will be required to coordinate the EIFS finish with the Construction Manager prior to ordering the material and installation.  Must be painted as described in plans even if using colored EIFS.

- Painting Contractor is to paint Storefront as shown on Storefront Rendering Elevation Plan. Include turnback on both sides above metal roof back to brick wall.

- Electrical Contractor is to coordinate with the Building Sign Installer for power disconnection/connection. New sign requires one 20amp circuit. EC is to remove conductors from all unused circuits to the closest junction box and remove from breakers. Circuit junction box for reuse should be mounted above the canopy, on the exterior wall, approximately center of the sign location. MC cable whip to the sign company’s wiring trough will be completed by the Electrical Contractor. The new sign is 24vDC and all wiring to the sign transformers is by the Electrical Contractor.

- General Contractor to pressure wash and brush scrub the standing seam metal roof.

- General Contractor to replace (5) Fire Lane signs (reflective), coordinate type with local governing authorities.  Remove the small Fire Marshall authority signs. Do not replace unless required by authorities. Fill holes with caulk to match brick/morter.

- Electrical Contractor is to replace (3) exterior receptacles.  New receptacles must have weatherproof cover.  Caulk new and existing receptacles.

- Electrical Contractor to remove electrical conduit and box on left side. Caulk holes with matching caulk.

- General Contractor to secure a screen to the sprinkler blow-off.

- General Contractor to remove (5) screws from brick wall on right side.

- General Contractor is to remove green plastic clips on transoms at both automatic doors.

- Painting Contractor is to repair front canopy ceiling around light fixtures.  Ceiling type is drywall.  Painting Contractor to paint front canopy ceiling, light fixture metal rings and access hatches.         (“Pure White” SW7005-Flat).   Electrical Contractor to replace (12) square light lens over front sidewalk.  Lens are National Account.

- Sprinkler Contractor to replace (14) sprinkler escutcheons.

- General Contractor is to caulk at sidewalk and around columns at storefront with “NP” type compound.  Especially needed at threshold of Automatic Door openings/\.

- General Contractor is to steam clean front sidewalks, storefront masonry, first two rows closet to the building of light pole bases, outside Produce Cooler at cooler/block transition and dumpster pads at end of project.  Must use hot water.  (Store Manager to sign off on completion).

- General Contractor is to have all storefront glass professionally cleaned after pressure washing the sidewalk at the end of the project. Make sure tape residue is removed on windows and doors. (Store Manager to sign off on completion).

- General Contractor to cut back wall attached “only” cart corral back to next wall support. Fill holes with matching caulk.

- Painting Contractor to prime and paint the existing cart corral SW 7068 "Grizzle Gray".

Front End/Vestibule Area

- General Contractor should contact Portico directly to coordinate installation date of the Vestibule Carpet.  Portico to remove existing VCT and mastic.  Contact is Bill Long - (877) 285-8899 x305 (or Steve Harrison x304).  NOTE: LEAVE A 5% OVERAGE FOR FUTURE USE.  THE STORE MANAGER MUST ACCEPT AND LOCATE.  The automatic doors must also be adjusted to allow for egress break away when installing the carpet tiles, if applicable.  Portico to provide new aluminum thresholds at all locations and anchor with flat head Stainless steel screws.

- General Contractor is to clean automatic door packages.  General Contractor to furnish and install a SS wrap (“U” shaped channel) on (8) doors 40” tall, not covering the locking mechanism.

- General Contractor is to clean (2) fly fans on the interior of the automatic doors.

- Painting Contractor is to paint the vestibule ceiling and access hatch.  General Contractor is to clean lights and diffusers.  Ceiling type is drywall. (“Pure white” SW 7005-Eg-shell)

- Painting Contractor to paint upper drywall walls. (“Roman Column” SW7562).

- General Contractor to remove (4) eyelet hooks in ceiling in the vestibule and make finish repairs.

- General Contractor is to remove poster hanging system from the vestibule windows.

- Electrical Contractor to remove (1) cover plate w/box, (2) cat5 and (1) outlet covered with red tape. Convert “clean” Quad receptacle to a “dirty” double receptacle. (The previous items are located under the existing Communicator Cabinets). Remove (2) other cat5 outlets in Front Wall.  Leave other (2) dirty quad outlets…make sure they have power to them.  Existing Outlet/cat5 to remain for relocated Coupon Kiosk. This work is in conjunction and addition to what is shown on Plans E4.02 and E4.03.

- General Contractor to relocate Front End Services as shown on A1.01.

- Painting Contractor is to prime/paint the 1x8 sill board under the (6) Vestibule windows. (“Macadamia” SW6142).

- General Contractor to reattach loose bronze brake metal window trim behind existing Primo Water.

- General Contractor to include tinting (6) Vestibule window(s) behind front end services located by the Construction Manager.  Tint is to be Llumar Window Film DL 05 GR SR CDF Gray.

- General Contractor must coordinate with the Store Manager on the night of the front end change out to insure there is adequate room to unload the front end equipment in the store.

- General Contractor will need to caulk new Belted Ends (closing all voids) to the floor after they have been properly installed, aligned and all of the VCT patch work has been completed. Use a color caulk that matches the bottom metals on the stands. (Appears to be silver/grey.)

- General Contractor to dispose of (4) security tiles (dark Plexiglas panels) and replace with ceiling tiles to match.

- Electrical Contractor is to provide a general use dirty power duplex receptacle where the Communicator Cabinet is shown relocated on plans for store communicators.  Locate receptacle behind cabinet, GC to cut out back of cabinet as needed for receptacle.

Outer Office/Office Storage

- Electrical Contractor is to remove old CR circuits at existing DSD counter, convert one duplex to dirty power, abandon and marlite over others.

- General Contractor and Electrical Contractor to provide demo as required of customer service counter and gate per print including the re-configuration of short wall.  Top out short wall with SS cap.  Coordinate with Construction Manager. Electrical Contractor is to remove any and all receptacles and communication cabling.  The existing communications CAT5 cables are to be labeled and pulled back to Communications Panel.  (Cut cables will be re-pulled at no cost to Retail Business Services.) Provide clean outlet and cat5 for relocated T&A cabinet.

- General Contractor to remove “Cody” marlite starting at the front corner of Customer Service wall to the Vestibule opening.  Install new “Cody” marlite/trim in this same area.  (end of Customer Service wall/do “Not” include outside wall, both sides of short wall by T&A, along wall where counter removed, Workroom wall w/door and safe, and then front wall to Vestibule opening.) See attachment under Important Notes in this document.

- General Contractor to install metal corners as necessary to protect corners on reconfigured short wall and adjacent full height corner.

- Electrical Contractor is to remove outlet behind clock over door. Painting Contractor is to make finish repairs before painting entire wall.

- Painting Contractor is to prime and paint angle (width of safe) on the floor behind the safe. (“Macadamia” SW6142).

- Painting Contractor to hand paint the office safe with a short nap roller, both inside the office and on sales area side. (“Macadamia” SW6142).

- Painting Contractor to paint both sides of door/trim and window trim. (“Macadamia” SW6142).

- General Contractor to remove counter and shelving behind safe in office storage and make finish repairs.  Existing Marlite to remain, install 1x fillers where shelving was removed. Painting Contractor to paint fillers, 2 x 6 with hangers and plywood base for wall installed cabinet to closely match color of Marlite.  Patch holes with matching caulk. Electrical Contractor to relocate (2) receptacle to 42" a.f.f. that will fall behind the new office cabinet.

- General Contractor to install the new Workroom Cabinet supplied by Retail Business Services with Front End equipment, caulk to the floor and wall.

- HVAC/Electric Contractor is to repair existing transfer fan.

- General Contractor to add a new louver kit in door.

- General Contractor to install a kickplate on both sides of the office door at the bottom of the door, also install a 12" kickplate behind the lockset above the louver on both sides of the office door.

Deli/Bakery Prep & Deli/Bakery Sales Area

- General Contractor to anchor the Hot Bar in place once floor tile work is complete by fastening an angle bracket to opposing sides at the base and to the floor under the hot bar.

- Electrical Contractor is to level up the existing telepole on the Hot Bar.  General Contractor is to replace ceiling tile and install new telepole trim.

- Electrical Contractor is to install uplighting on high cases #1 and #2 under bulkhead. National Account.

- General Contractor is to install kick down door stop on Deli Closet door.

- General Contractor is to remove and dispose of 8’ Bread Workstation and fillers.

- General Contractor is to construct a partial height stud wall as shown in plans (See A3.01, Detail 1). Repair/fill in Quarry Tile as necessary and install QT Base as shown in plans (See A3.01, Detail 7).

- Note: Do not add 4’ LED Fixture/Circuit for TOI on bulkhead.

- Electrical Contractor to provide a dirty power dedicated circuit to a junction box above the ceiling above Cases #3 & #4 as shown on A1.01 drawing for the LED Menu Monitors (circuit cannot be tied to the Energy Management Panel and must have continuous power).  Once the monitors are installed by others, install a duplex flush mounted in the ceiling tile within 6" of the monitor post and run BX Cable from the JB to the receptacle.

- General Contractor to thoroughly clean the deli in the area where the new half wall was added once the work is complete.

- Painting Contractor to paint Deli Closet door/frame inside and out.  (“Macadamia” SW6142 – Pro Industrial Pre-Catalyzed water based semi-gloss Epoxy)

- Electrical Contractor is to provide a 110v Circuit for infrastructure for future Breading Station.

- General Contractor must verify that the Deli sales area bulkheads are structurally sound to support additional wall loads. Refer to detail in plans for structural repairs if needed.

- General Contractor must submit a letter to Retail Business Services Construction that the existing/remaining bulkheads have been certified that they meet structural and architectural criteria.

Produce Sales Area

- Note: Existing trellis is to remain. Do not paint.

- Note: Uplighting is to remain.

- Electrical Contractor is to install 16’ of lay in lighting for Décor as shown on Plan E1.01. Extend another 8’ not shown on Plan towards Produce Prep Doors.  Install 20’ centered over “Produce Décor” as shown on Plan E1.01.   National Account.

Produce Prep

- General Contractor is to install new “Black” nylon double swing doors into the sales area and into backroom.

- General Contractor to remove window insert from a double swing door being removed from Restroom Corridor and replace broken insert going into Grocery Storage.

- General Contractor is to remove existing Humidifier system in preparation for new Misting System. Note: Vendor to install polyboard.

- Plumbing Contractor to supply ½" cold water with ball valve for the misting system as located by the Construction Manager.

- Electrical Contractor to provide three (3) 20amp dedicated GFI Receptacle for the Misting system as located by the Construction Manager.

- General Contractor to maintain existing Flying Insect Traps (with Glue Boards) in the Produce Prep.  Remove temporarily for finishes, do not discard.  Vendor (through Food Safety Dept.) will be responsible for replacing units as needed as part of the “Fly Program”.

- General Contractor to pressure wash the Produce Prep and Cooler floor.  General Contractor to also chase 14 linear feet of cracked concrete and fill with two part epoxy.

- General Contractor to construct a chase wall where new Prep Sink is being installed. (See A3.01, Detail 2).

- Plumbing Contractor is to install new Prep Sink.

Meat Prep & Meat Sales area

- General Contractor is to install new “Black” nylon single swing door into sales area.

- General Contractor must wipe down and clean the electrical cord drops from the ceiling.  Zip Tie all un-used cord drops to the ceiling.

- General Contractor to remove abandoned data wiring to scales and seal holes in ceiling.

- General Contractor is to cut out and remove existing loose caulking along top of all perimeter quarry tile cove base where necessary.  Apply new caulk (color to match grout) to seal the quarry tile cove base to the wall where needed.

Sales Area

- General Contractor is to ensure that all trades cleanup after their work has ended for the day or prior to them leaving the area.  This is to include removal of debris along with any needed dusting, sweeping or mopping.

- General Contractor shall replace all HVAC return grills on sales floor and in deli.

- General Contractor is to clean all sales area diffusers.  Replace (40) ceiling tile that are stained/damaged with matching ceiling tile.

- General Contractor is to leave 20 ceiling tiles that will be used for Loss Prevention for security cameras, etc.  Ceiling tiles are to match the sales area ceiling tile, place them on the storage platform in the backroom.

- General Contractor must remove all unused clips from the ceiling grid.

- General Contractor is to re-secure/reattach and provide sprinkler escutcheons as required throughout the store.

- General/Painting Contractor is to remove all Décor including wood trimmed décor signage and other vertical wood features. Make finish repairs.

- Painting Contractor is to skim the sheetrock walls where old décor is removed and achieve a “Level 4” finish prior to painting.  PC is to skim all imperfections in all sales area drywall as necessary prior to painting the sales floor walls.

- Painting Contractor to paint walls, columns and round chases over FF/IC. (“Roman Column” SW7562).  1’5” Perimeter Wall Band (“Meditative” SW6227 – Latex Egg-Shell)

- Painting Contractor to refer back to the “Standard Notes” section of these bid notes for directions concerning the National Account program.

- Painting Contractor is to paint all backwall door frames. (“Macadamia” SW6142 – Pro Industrial Pre-Catalyzed water based semi-gloss Epoxy)

- General Contractor to install drywall control joint and repair drywall crack located at each end of front vestibule windows.

- General Contractor is responsible for installing new Fire Extinguisher stickers on all sales area and office Fire Extinguishers once all painting and wall finishes are complete.  New stickers are to be as follows:

  - Fire Extinguisher Signs for Curved Surfaces (including round and square columns) – Provide 18"H x 4"W glow-in-the-dark adhesive sign equal to Condor 469V23 (Grainger item #469V23).

  - Fire Extinguisher Signs for Flat Wall Surfaces – Provide 18"H x 4"W glow-in-the-dark plastic sign equal to Accuform Signs MFXG551GP (Grainger item #9CXE3).

- General Contractor is to include the material and installation of all Finish Metal work required.  Acceptable standards are:

  - 18 gauge Stainless Steel, #3 Finish, non-hemmed edges (PVC coated) on all exposed corners across the storefront, boxed column wraps are to be 22 gauge, #3 Finish Stainless Steel.

  - 11 gauge Stainless Steel, #2B Finish (PVC coated) thresholds at all Deli openings, Meat Prep to Sales Area opening, Produce Prep to Sales Area opening, and any Sales Area to backroom opening.

  - 18 gauge, #3 Finish, Stainless Steel with non-hemmed edges (PVC coated) 4' corners on all outside Deli Hard Tile corners.

  - 20 gauge, #3 Finish Stainless Steel (PVC coated) for all Oven and Hood Flashing.

  - 18 gauge, #3 Finish Stainless Steel (PVC coated) for all Kickplates.

  - 18 gauge, #3 Finish Stainless Steel (PVC coated) 4’ corner guards on all sales area exit door openings, Produce Prep door opening, and Meat Prep Door opening.

  - 18 gauge, #3 Finish Stainless Steel (PVC coated) Restroom Corridor opening wrap that wraps 2” on the sales area side, through the opening and wraps 2” on the Corridor side to create a full cap.

  - 18 gauge, #3 Finish Stainless Steel (PVC coated) 4’ corner guards on any Restroom Corridor exit door openings, restroom door openings, backroom traffic door opening, and any other Restroom Corridor door openings, include an 18 gauge, #3 Finish, Stainless Steel with non-hemmed edge 4’ corner on any outside wall corner in corridor.

- General Contractor is responsible for demo of existing fillers and wedges shown to be removed prior to the Décor Vendor's arrival and case painting.

- General Contractor shall be responsible for removing and disposing of the old valence to include brackets from the cases.

- General Contractor is to install plywood caps over all gaps between shelving the length of the shelving and in gaps between angled cases.

- General Contractor to remove sign headers from Lunch Meat, Dairy, and Beer cases.

- General Contractor to remove debris and broom sweep tops of all cases at project completion.

- Soft Tile Contractor to remove 1 layer of VCT prior to new floor patch tile installation.

- Soft Tile Contractor shall patch and repair floors due to Belt end change-outs (75) tile, Customer Service Low Wall Removal (10) tile, where Breadstation was removed in Deli and new low wall has been constructed (12) tile, existing damaged VCT including (2) dark tile between last checkout and Deli and (10) tile in front of Double Door going into Restroom Corridor-(Note: Also install metal transition strip at Restroom Corridor Dbl. Door).  There is 112 estimated tile.

- General Contractor shall be responsible for installing McCue corner guards as shown on the fixture plan.

- General Contractor to wipe clean all sales area emergency lights and exit signs prior to the completion of the project after all major sales area work is complete.

Restroom Corridor

- General Contractor is to install new “Black” nylon double swing doors into Restroom Corridor from Sales area.

- General Contractor is to install new Black ADA approved restroom placards beside doors.

Remove existing placards from doors.

- General Contractor to remove the marquee poster and frame, make finish repairs.

- Painting Contractor to paint CMU walls around inside swing door opening, along Restrooms and Lounge wall, along back wall down to partition and then the partition wall (“Roman Column” SW7562). All doors and door frames along this same area to be painted (“Macadamia” SW6142).

Note: Dairy Cooler (2) walls will be Electrostatic Painted (“Macadamia” SW6142).

Restrooms

- General Contractor to adjust both doors for ease of closure. Install kick-down door stops on both.

- General Contractor is to clean drywall restroom ceilings.

- General Contractor to clean the exhaust fan grills in both restrooms. Need to install a white 14’ x 14” grill in the Women’s Restroom.

- Painting Contractor is to paint inside doors and frames (White).

- General Contractor to clean the sinks, urinal, and toilets with pumice stone to remove hard water stains. Re-caulk all fixtures as necessary.

- General Contractor is to replace both sets of restroom toilet partitions (color to be stainless steel).  Urinal screen is to have overhead bracing. (Note: legs of new partitions must not fall on top of floor drain.) Partitions must be made by one of the following manufacturers:  American Sanitary Partition Corporation, Global Steel Products Corp, or Metpar Corp.

- General Contractor to install 2 coat hooks in each restroom stall, one mounted at 48" and the other mounted at 59".  Coat hooks are to be secured with thru bolts (tamper resistant Pin-In-Star on inside and smooth head barrel style binding post on outside “with a dab of Loctite Threadlocker”).

- General Contractor is to replace restroom mirrors with (1) standard style in Men’s Room.

- General Contractor is to fill any holes with matching caulk in tile where old partitions removed.

- General Contractor must contact and use Rick Ling (Ph # 336-918-2231 or email: (rling@newlifetilecleaner.com) with New Life Tile for the installation of the sealer used in the restrooms. This shall include all labor and material associated with the sealer installation.

- General Contractor is to professionally clean all restrooms, complete this work at end of project and must have the Store Manager to sign off on the completion.

##### Lounge

- General Contractor to remove all items from existing wall paneling (including tape residue) and make paint ready.  Painting Contractor to paint the existing wall paneling, refer to "Standard Note" section for acceptable product and procedures. (“Macadamia” SW6142).

- General Contractor is to lower the existing lockers and mount to wall 8” AFF.  Provide a sloped  canopy on top of the lounge lockers and paint.

- General Contractor to remove corner shelves and make repairs.

- Painting Contractor is to paint existing wood coat rack. (“Macadamia” SW6142).

- Painting Contractor is to paint both sides of door/frame and window frame. (“Macadamia” SW6142).

- General Contractor to adjust door where catching on frame and install a kick down door stop.

- Electrical Contractor to install receptacles for coffee and microwave 50" a.f.f.

- Electrical Contractor to install a duplex outlet for TV Monitor 15” down from ceiling and center the refrigerator width. A dedicated circuit is not required.

##### Grocery Storage Area

- General Contractor is to remove the construction debris from the tops of all walk-ins and rooms and sweep clean.

- Electrical Contractor to provide power for the (2) new bug zappers in the backroom.  Do Not remove, relocate, unplug, or discard the Flying Insect Traps (with Glue Boards).

- General Contractor to assemble and install:  Recycle bins, misc. tables, ALL trashcans, lounge furniture, office furniture, etc.  Coordinate with Construction Manager.

- Tile/General Contractor to install 2” min. flat stock aluminum threshold at all VCT transitions.

- Painting Contractor to include painting the white rodent stripe on the perimeter of the backroom storage area. This will “not” include areas behind large shelving bunkers. NOTE: Store Manager is to have a (2) weeks notice before beginning.

- Painting Contractor is to paint all doors and door frames. (“Macadamia” SW6142).

- General Contractor to install Safety Rail Kits to the tops of all walk-ins and rooms as require.

##### Mechanical Room

- General Contractor to install new passage latch door handle on the motor room. (No lock)

- Electrical Contractor to add a general use receptacle to the left of the roof access ladder.

##### Janitor’s Room

- General Contractor is to re-secure the janitors closet ceiling. (1 pc plywood).

- Painting Contractor is to paint door frame both sides. (“Macadamia” SW6142).

- General Contractor is to clean the exhaust fan grille.

- General Contractor is to provide a minimum of 4 neoprene spring operated mop hangers mounted above the can wash.

- General Contractor is to re-grout (25) pieces of quarry tile in the janitor can wash floor.  General Contractor is to clean all tile.

- Plumbing Contractor to install new eye wash. (No longer supplies the mixing valve for the Eye Wash Station, this will be part of the equipment supplied).

##### Department Manager’s Office/PC Training Room

- General Contractor is to adjust sprinkler head in existing sheetrock ceiling and make finish repairs.

Install escutcheon where missing.

- Painting Contractor is to paint the existing sheetrock ceiling.  (“Pure White” SW7005)

- Mechanical Contractor to install a new transfer fan, coordinate the location of the transfer fan with the Construction Manager. Electrical Contractor to provide power.

- Paint Contractor to paint the existing block walls, include painting exposed conduit and junction boxes.  (“Macadamia” SW6142).

- Paint Contractor to paint door/frame both sides.  (“Macadamia” SW6142).

- General Contractor is to provide and install a 10’ laminated millwork counter top.  Coordinate utilities with counter location!  Counter to be 30" above finished floor.  Construction Manager to provide millwork details.  (Laminate: “Desert Beige” #899-58)

- General Contractor to provide plastic grommets for wiring penetrations of countertop.

- General Contractor to install a 10’ laminated millwork shelf mounted 28” above countertop.  Laminate to match counter.

- Electrical Contractor to install one quad receptacle 64” AFF over the shelf.

- General Contractor is to construct an electrical chase. (See A3.01, Detail 4).

- General Contractor is to construct a chase for existing refrigeration lines.

- Electrical Contractor is to install circuits to one single vertical chase located in the back right corner as shown in plans and run horizontal along existing CMU to finished boxes for all DMO/PC Training electrical.

- General Contractor to install push/pull plate hardware on door and a kick down door stop.

- Soft Tile Contractor is to install VCT (“Earthstone Greige” 12” x12” tiles in stacked pattern). Install 4” black vinyl base.  Install 2” min. flat stock aluminum threshold at all VCT transition.

Rear Exterior & Sides

- General Contractor is to properly re-stripe truck guide stripes (as per Food Lion Plans and Specifications).  Provide glass beads for stripping.

- General Contractor is to clean the light fixtures over the dock doors.

- General Contractor is to install approximately (7) concrete splash guards around the rear of the store over the asphalt.  Splashguards are to be silicone to the asphalt and silicone around edges to hold its position.

- General Contractor is to re-stencil the store numbers on the back dock doors and roll-up doors.

- Painting Contractor must repaint exterior metals to include stairs and rails, doors, doorframes, bollards. Use similar color as existing.

- General Contractor to re-install (1) loose bollard that is leaning located by DSD door.

---------------------------------------------------------------------------------------------------------------------------------------------

###### IMPORTANT  NOTES

---------------------------------------------------------------------------------------------------------------------------------------------

---------------------------------------------------------------------------------------------------------------------------------------------

- Subject: FL1662 Health Comments

Hi Jeff,

The other comment for 1662 is “ Be advised that all the proposed relocated existing equipment for this project will be further evaluated during the remodel site inspection to determine whether the equipment is clean, serviceable and operational to the satisfaction of this Department. This office will require that you agree to repair or replace any relocated equipment if it is found to be in an unsatisfactory condition.”

Just wanted to let you know, so your construction team knows.

Thanks,

-Jennifer

| Jennifer McKinney, RA, NCARB |  |  |  |  |
| --- | --- | --- | --- | --- |
| Project Manager, Architectural Services |  |  |  |  |
| Office: 407.645.5008 |  |  |  |  |
| 604 Courtland St, Suite 100 ¦ Orlando, FL 32804 |  |  |  |  |
| Orlando · Chicago · Dallas/Fort Worth · Philadelphia |  |  |  |  |
| Architecture · Engineering · Permitting · Interior Design |  |  |  |  |
| jmckinney@interplanllc.com ¦ www.interplanllc.com |  |  |  |  |
| AA 003420 ¦ CA 8660 ____________________________________________________________________________________________________________ I can submit a general list of guidelines (as follows) for construction work but the detailed construction plan should be provided by the selected GC as required. General Guidelines for Construction Work in Prep Areas - All construction work in these designated areas are typically scheduled when the store is closed. - Construction methods typically include floor to ceiling plastic dust walls to protect non-construction areas. - All food prep areas/surfaces are cleaned according to mandated health regulations prior to opening hours. - All food items are stored in closed display cases and refrigerated coolers and freezers during non-business hours. - All cooking devices including ovens, fryers retarders, etc are cleaned and sealed prior to store closure. General Guidelines for Construction Work in Restrooms - If both restrooms are to receive an upfit, only (1) restroom at a time is scheduled for closure and construction activities during the course of the remodel. The RBS Construction Manager, Alan Wilson, may be contacted (as follows) for the construction phase of the Remodel. Alan Wilson Construction Manager 1 Retail Business Services, LLC c: 704-202-3310 e: alan.wilson@retailbusinessservices.com Hopefully this answers most if not all of the reviewers questions. Let me know if you need anything else. Best regards, Jeff Maness Engineering Project Manager (t) 704-310-2731 (f) 704-645-2076 (e) jeffrey.maness@retailbusinessservices.com - From: Emily Cloud <ECloud@interplanllc.com> Sent: Thursday, February 20, 2020 9:59 AM To: Jeffrey Maness <Jeffrey.Maness@retailbusinessservices.com> Cc: Jennifer McKinney <JMcKinney@interplanllc.com>; Donna Brown <DBrown@interplanllc.com> Subject: 2019.0893 FL1662 Health comments Good Morning, We received Comments from Health for Food Lion 1662, in which we need your detailed schedule of construction. The Comment: - Specify whether the facility plans to close or remain in operation during this remodel; The scope of the remodel may not allow the Deli Bakery Prep area, the Produce Prep area, and the restrooms to remain open during some portions as specified. If facility intends to continue to operate the aforementioned areas during the remodel, a detailed schedule of the phased construction is to be submitted. This must detail what precautions will be used for the protection of patrons, the protection of product in the existing facility that will be impacted during construction, and the protection of food/food contact areas adjacent. Access to a restroom inside the facility, potable hot/cold running water under adequate pressure, and uninterrupted flow of wastewater are also requirements for the aforementioned areas to be in operation. A loss of any preceding utilities while in operation may be grounds for closure; The detailed phased construction plan must detail all stages of the remodel, precautions in place to protect food and the post construction cleaning and sanitizing that is to occur after. The Prince George’s County Health Department may assign an inspector to coordinate and ensure compliance to what is proposed in the resubmission. Please let us know if you have any questions. Thank you, Emily Cloud Emily Cloud. Project Coordinator, Architectural Services. Office: 407.645.5008. 604 Courtland St, Suite 100 ¦ Orlando, FL 32804. Orlando · Chicago · Dallas/Fort Worth · Philadelphia. Architecture · Engineering · Permitting · Interior Design. ecloud@interplanllc.com ¦ www.interplanllc.com. AA 003420 ¦ CA 8660. |

---------------------------------------------------------------------------------------------------------------------------------------------

###### Allowances

---------------------------------------------------------------------------------------------------------------------------------------------

*Note: General Contractor is to incorporate in this contract the listed allowance as stated in the bid notes per the Retail Business Services Construction Manager.

---------------------------------------------------------------------------------------------------------------------------------------------

  - None.

CC:	Construction Manager – Alan Wilson

General Contractor – Wimco, STH, Windle, Vertex

Energy Team Member – Sammy Mickel

Construction Coordinator – Angie Wingler

AW/afw

1662walk