## FOOD LION TAKEOVER BID WALK NOTES

## STORE #___2250	Address: _1555 E. Union Blvd. Morganton, NC

____________________________________________________________________________

*Important note:

ALL attached bid notes are specific and will override that specific instance located in the drawings. Carefully read bid notes and review drawings to accurately bid this job. Any deviations unclear should be questioned for accurate bid and build. Food Lion will not accept responsibility for un-bid items indicated in notes or drawings due to improper distribution of available information to the General Contractors respective subcontractors. General Contractor is responsible for the timely distribution of updated information to ALL sub’s and field personnel.

*___________________________________________________________________________

Standard Notes:

- The bid date for this project is February 4th, at 2:00 p.m.  This project will be bid through Delhaize America’s Project Management Platform PM Web.  For questions or support contact: Shana Hines at 704.310.4210 or Shana.hines@retailbusinessservices.com

2.	All bids must contain name of plumbing, electrical, roofing and HVAC contractor and their bid price.  A minimum of 3 bids are required per trade.  A copy of the bid cover page is required.

- Food Lion will provide Builders Risk and Owners Protective Liability Insurance.  The contractor must provide liability insurance.

- Contractor shall include a payment and performance bond to be priced as an alternate. The bonding company’s A M Best Rating must be B+6 to A-6.  Proof of this rating must accompany bond.  Bond rating MUST be within the past 30 days.

- INCLUDE THE PERMIT COSTS IN YOUR BID.  -  The Architects will submit the drawings for permitting, but the General Contractor is responsible for picking up and paying for the permit.

- Landlord Information:  (Leased or Food Lion Owned)

Landlord’s contact info.Grunkhorn Attn:Judy McCormack, Morganton, SSG, LLC

Address: PO Box 778

Armonk, NY 10504

- The project specifications are specific to this project. These specs and associated addendums will govern all workmanship and materials. The General Contractor will be required to provide the bidding sub-contractors a copy of the store specifications which will be adhered to during construction. Do not reference any other Food Lion specifications other than the manual established for this project. Any questions or discrepancies must be confirmed through the Construction Manager. These specs can be accessed through ProTrac.

- General Contractor shall provide unit pricing in bid proposal for the following items:

  - Rock excavation (Cubic yard).

  - Finished concrete (cubic yard).

  - Bad soil removal and replacement (cubic yards).

  - Asphalt removal and replacement (square yard).

  - Square footage cost to replace bad roof decking on projects that require roof replacements.

  - Concrete slab demolition and removal.

- General Contractor shall video tape or photograph existing exterior conditions at store prior to any work beginning.  This should especially include the conditions of the roof and asphalt drives and parking areas.  Any back charges by the landlord that cannot be proven false by existing photographs will be paid by the General Contractor.

- Contractor MUST maintain an OSHA approved safety program.

- The awarded General Contractor must submit a “Schedule of Values” for this project when he submits his contract to Clarence Ballard for his review and signature.

- The awarded General Contractor is required to submit a partial Release of Lien, equal to the amount of the pay request, to accompany each Contractor’s Application for Payment.  Payment will not be made until each pay request has this information attached.

- The General Contractor must incorporate the Work Flow billing procedures as set up by Food Lion. Please contact Lori Bowen (704-310-3036) for any questions or concerns related to billing.

- Food Lion reserves the right to accept any bid or bid alternate, to reject any or all bids, or to waive any informalities in bids received where such acceptance, rejections, or waiver is considered to be in the best interest of Food Lion.

- Drawings for Delhaize America projects are housed on the company’s Project Management Site, PM Web. Each General Contractor is responsible for downloading their drawings (along with Refrigeration Summary, Asbestos Survey, Roof Survey and Delhaize America Specifications) . There is no fee to download drawings from the site. For questions or support contact: Shana Hines at 704.310.4218 or Shana.Hines@retailbusinessservices.com

- General Contractor must ensure that all subs related to the takeover store receive a copy of the refrigeration summary, asbestos survey, Food Lion Specification Book, etc. so the sub can research and incorporate a price appropriately for the work to include labor and material.

- The existing ceiling tiles must be recycled as part of Food Lion's sustainability program. An approved ceiling tile contractor is:  Southeastern Interiors.  Contact: Jerry Milton, Phone #: 910-893-8486, Cell #: 919-427-8965.

- Fiberglass pits must be furnished and installed as indicated in the plans. The concrete caps on top of these pits shall be finished flush with a minimum 2” concrete to the finished slab, allow for plywood bottom split in half to allow for future pit access.  All revised pits must be sealed completely to rodent proof and eliminate construction material from damaging the refrigeration lines. Special precautions need to be made when cutting holes in fiberglass boxes that they limit the amount of fiberglass that is cut away for line installations.  All pits shall be fiberglass.

- Electrical Contractor is to ensure that no MC cables are used on a vertical or horizontal location for receptacles or to emergency lights; MC or flex cable can only be used above the lay-in ceiling and strapped to bar joist.  EMT must be used to connect panel to junction box.

- Electrical Contractor must install wire guards over lights and sprinkler heads on 8’-0” ceilings or lower throughout the entire store.

- The general contractor is responsible for protecting the VCT once this is installed. VCT laid in sales area shall be treated prior to any foot traffic with 2 coat application - (1) coat Armstrong S-495 Sealer and (1) coat Armstrong S-480 Polish.

- Flooring Contractor must meet Food Lion’s plans and specifications for installation of VCT. This shall include all necessary prep work to include proper testing, floating, sanding, and prepping of the floor prior to VCT installation. The Flooring Contractor must coordinate work through the Construction Manager and General Contractor. Moisture tests are the responsibility of the flooring contractor and to be overseen and verified by the General Contractor. All flooring materials (patching compounds, adhesives, moisture barrier, self-leveling underlayment, etc.) will be provided through MAPEI. See specification book for details.

- General Contractor is responsible for hanging Grand Opening banners supplied by the Market Specialist. Construction Manager will indicate how and where these will be hung.

- General Contractor must have the punch list items complete (Both General and Sub-Contractors) within three weeks of receiving it. If not, Food Lion reserves the right to augment the workforce and complete the work at the General Contractors cost.

- General Contractor’s Project Manager is required to meet with the Construction Manager on a weekly basis on the job site.  Meeting times to be determined by the Construction Manager.

- General Contractor is responsible for verifying, obtaining, and incorporating the most current fixture plan and distributing it to the subs. Failure to obtain the most current fixture plan will result in the General Contractor having to make the necessary changes at their cost.  The most current fixture plan is always available through PM Web.

- General Contractor and Sub-Contractors are responsible for completing all SESCO punch-list items and notifying SESCO that the punchlist items are complete and the project is ready for reinspection. SESCO will set a punch list date based on the store opening date.

- General Contractor must use PEMKO #18100CNB84 bristled type sweeps on exterior doors; rubber gasket sweeps are no longer permitted. (Black)

- General Contractor is to install weather-stripping, sweeps and new style carpet thresholds on all exterior doors. (This includes Automatic doors.)

- National Account items per the drawings will be purchased by Food Lion and installed by the General Contractor. The general contractor will complete the National Account order form and return it to Food Lion once all national account orders are placed.

- The new concrete slab will be tested for flatness and levelness.  The General Contractor shall contact Crescent Construction Services to schedule this service a minimum of 2 weeks prior to test date.

- Plumbing Contractor is to install snap-in perforated drain covers in all exposed drains.

- General Contractor is to install door silencers on all doors.

- General Contractor must install commercial grade kick downs on all interior hollow metal doors.

- General Contractor is to install stainless steel kick plates on all doors.

- General Contractor shall caulk around all fixtures in restrooms/all corridors/produce prep/meat prep and deli/bakery area and all offices throughout the entire store – and/or accessories, which fasten to the wall.  (Examples:  water fountains, sinks, paper towel holders, electrical plate covers, plumbing fixtures, plumbing escutcheons, electrical outlets, sprinkler escutcheons, wall mounted fixtures, etc.)

- The freezer door switches will be provided by the mechanical supplier to the Refrigeration Contractor.  It will be the Electrical Contractor’s responsibility to mount the sensor and provide conduit and wiring back to the rack for alarming.  Final terminations will be by RC.  The Electrical Contractor shall also provide wiring conduit and wiring from freezer door switch to rack control panel.

- General Contractor will be responsible for “ALL” dumpster dumps including those for the Refrigeration Contractor and grocery set.  A Food Lion Vendor will be responsible for removal of shelving metals, General Contractor will be responsible for all other discarded material during grocery set.

- Food Lion will supply the Automatic Doors, Dock doors, Dock Levelers, Dock Seals, Electrical Switchgear and panels, Control Panels, Interior Lighting, Impact Doors, Sinks, Water Fountains, Water Heaters, Vestibule carpet,  Aqua stat & Stands, Hand Dryers and HVAC equipment.  Installation and warranties of this equipment is to be supplied by the General Contractor.  The General Contractor is responsible for ensuring that the SUB-CONTRACTORS  order  these items directly from the supplier as soon as the contract is awarded.  These items must be stored on site in the Sub-Contractors storage trailers.  A complete package with all the information will be sent to the awarded General Contractor.  National Account items MUST BE EXCLUDED from your bid, with the exception of installation and warranty.  General Contractor must show that ALL orders have been made with deliver dates at the Pre-Construction Meeting by submitting the National Account Order Form to the Construction Manager.

Automatic Doors – Assa-Abloy / Besam – Carolyn Brown @ (866) 640-9667, ext. 120 
Email:  Carolyn.Brown@assaabloy.com

Power Wall-Electrical Switchgear  – CD Controls – Contact: Ed Rios @ 706-476-7303 (Cell), Email:  edrios@cdcontrols.com

HVAC System/Energy Management/ Environmental Control – Sesco/McNamara – Contact: Sharon Turner/Penny Shepherd @ 336-996-2220, Email:  Sharon@trs-sesco.com

Interior Lighting Package – Sylvania Lighting – Contact: Lisa Mastin @ 1-800-579-1514

Email: lisab.mastin@sylvania.com

Hollow Metal Doors & Hardware – B.A. Hoft – Contact: Andy Gibson @ 919-596-4446
Email:  agibson@bahoft.com

Dock Doors, Seals & Dock Leveler – Overhead Door – Contact: Devin Mooney 
@ 469-549-7025, Email:  Devin_Mooney@OverheadDoor.com

Traffic Doors – Mueller – Contact info to follow.

Vestibule Carpet – Portico – Contact:  Greg Griffin @ 877-285-8899

Email:  ggriffin@porticosystems.com

Hand Dryers – George Scott & Associates – Contact: Bill Scott @ 704-376-4775

Email:  bill.scott@gsaclt.com

Sinks – John Boos – Contact: Michelle Harrelson @ 800-327-1817

Email:  cs1@hefcollc.com

Water Fountains, Water Heaters, Stands, and Aquastat (Therma-Stor) – WW Grainger  
Contact:  Caleb Lyall @ 704-501-0150, Email:  orders.128@grainger.com

NOTE:  General Contractor to provide a list in each bid of the National Account items Food Lion is to supply.

- Electrical Contractor to provide a commercial qualified Lead Electrician for this project.  This person is to be identified as the Project Electrician and is to work on site a minimum of 40 hours each week starting from week 1 of the Construction Schedule to 100% complete.

- Electrical Contractor shall install all vertical low voltage cabling in finished and unfinished areas in EMT with a 90° bend at the bar joist in the direction of the cable run.

- Electrical Contractor shall install all low voltage cabling running overhead in the bar joist and tied every 15 ft.  Cabling that is suspended from the bar joist or lying on a ceiling will not be accepted.  Electrical Contractor is to work with each Food Lion vendor to remove any and all abandoned low voltage cabling.

- The refrigeration contractor will install the front-end equipment; this includes but is not limited to:  all check stands, cigarette kiosk, and Customer Service counters.  The refrigeration contractor will also caulk these fixtures to the floor. The electrician will install and terminate all clean and dirty power in this area. All communication cabling will be installed by Macro when they arrive at the store.

- Electrical Contractor is not allowed to use MC (flex) cable for electrical runs.  EMT must be used to connect panel to junction box.  MC (flex) cable is only allowed as fixture whips or extensions from a junction box above sales area ceiling to cases/equipment below. Sealtite conduit must be used in any potentially wet areas.

- General Contractor must use Contract Flooring (contact #800-356-6792 or #252-522-4588); Lockwood Carpets (contact #804-693-2372 or #804-824-4200), Haywood Floor Covering, Inc. (contact #804-824-9741 or #804-815-2295), Tino Tile (contact #704-636-7735), or Inside Edge Commercial Interior Services, LLC (contact #651-389-3938 or #636-219-3675) for ALL store soft tile installation.  No other soft tile contractors will be accepted.

- Any new cable installed will be a Belden 1212 CMR Cat5e 4 pair 24 AWG (Belden 1200 DataTwist Cable Series).

- All telepoles in the sales area are to be anodized aluminum. This includes, but is not limited to the cash registers, customer service kiosk, produce and deli bakery sales area. Confirm the quantity of telepoles with the Construction Manager.

- The general contractor will seal the joint between the coolers/freezers to any adjacent walls. Also, seal any existing cracks, crevices, holes, etc. in the existing masonry walls prior to coolers and freezers being set.

Safety Notes:

- General Contractor MUST maintain an OSHA approved safety program. A copy of this safety program should be provided to Food Lion.

- Electrical Contractor will not be allowed to work on live electrical loads.

- Tie-off and fall protection should be used at all times in compliance with the general contractors approved safety program.

- Food Lion should be copied on any OSHA inspection reports that take place on the site.

- General Contractor is to follow the following guidelines as a minimum when Propane powered equipment is used inside the store:  (NO GAS POWERED EQUIPMENT)

  - Personal Carbon Monoxide Measuring Device is to be worn during operation of emission generating equipment.

  - Ventilation to be reviewed by Construction Manager prior to operating.

---------------------------------------------------------------------------------------------------------------------

The following are department specific notes, please read thoroughly and coordinate with other store documents.

---------------------------------------------------------------------------------------------------------------------

GENERAL NOTES

- General Contractor is to install burglar bars on all new and unused existing curbs larger than 12” on roof.

- General Contractor is to install a door scope to driver’s/DSD door.  (Beeson Hardware 336-821-2181).

- General Contractor/Sprinkler Contractor is responsible for the fire sprinkler line & riser to be in good working order and relocate sprinkler lines to the new layout and exposed sales area ceiling.

- General Contractor is to install an insulated floor under all walk-in freezers. See the plans for specific dimensions and details.

- General Contractor to contact Crescent Construction Services to conduct levelness and flatness testing for the entire floor slab (excluding prep areas). Food Lion will absorb the cost for 2 inspection trips to the store by CCS. All additional trips will be the responsibility of the general contractor at $1,200 per trip.

- General Contractor to contact Crescent Construction Services for inspection of final sanitary sewer lines. Consult with Crescent and Food Lion Construction Manager on any corrections required prior to pouring any floors. Food Lion will pay the cost for 2 sanitary sewer inspections at the store by CCS. All additional trips will be the responsibility of the general contractor at $800 per trip.

- General Contractor to contact Crescent Construction Services to conduct release to pour inspections prior to the slab being poured. Food Lion will pay for 2 trips to the store by CCS. All additional trips to the store will be the responsibility of the general contractor at $800 per trip.

- General Contractor to contact Crescent Construction Services to conduct final electrical inspections. Food Lion will pay for 3 of these inspections. All additional trips will be the responsibility of the general contractor at $800 per trip.

- General Contractor to include the application of 30% solid concrete sealer to the exposed concrete floor in the Produce Prep Room, Produce Cooler, Stock Room, and Dairy Cooler.  Refer to Food Lion spec book for the material to be used and application- minimum of two coats.

- General Contractor is to install paper towel dispensers provided by Food Lion.

- General Contractor must install conduit sleeves over threaded rods at all boat racks hanging from the ceiling.

- General Contractor is responsible for the temporary power and lighting to the store.

- Electrical Contractor is to install conduit and junction box for the microphone at DSD, & Dept Manager office, etc.

- Electrical Contractor is to install conduit and wall boxes for Muzak volume control.

- No surface mounted conduit or boxes will be permitted on the sales area walls.

- Electrical Contractor shall install dated and type written panel schedules in all electrical panels, new and existing, at the end of the project.  Electrical Contractor is responsible for new schedules whether Food Lion provides Power Wall or purchased by EC.

- Handwritten panel schedules shall be maintained throughout the project.  Electrical Contractor shall label the exterior of the electrical panels with phenolic labels.

- Electrical Contractor must install wire guards over lights on 8’-0” ceilings or lower throughout the entire store.

- Plumbing Contractor must install a 4” bell hub on all exposed hub drains to eliminate spill over.

- Plumbing Contractor will be responsible for running PVC drain line from case drains to their respective hub drains (Required on new stores and take-overs).

- Plumbing Contractor must run individual condensate drain lines from all evaporators. NO condensate drain lines can be tee’d together.  ALL evaporator condensate lines are to be anchored to the wall and hanger supports from the ceiling. Install plumbing escutcheons on wall penetrations.

- Plumbing Contractor will not use Cellular Core PVC anywhere on this project.

- Painting Contractor is to install a 2”x2” steel angle (width of safe) on the floor behind the safe in the office. Anchor to the floor after positioning safe.  Angle to be primed and painted.

- Painting Contractor must repaint all exterior metals to include roof top steel, stairs, canopy supports, doors, doorframes, cart corral on sidewalk, bollards, shopping center signs, light pole bases, etc.

- General Contractor must have the Sprinkler Contractor to install white semi-recessed sprinkler heads in rooms that get new ceilings/coolers/freezers. This shall include all labor and material. All sprinkler escutcheons throughout the entire store should be secured to the ceiling. All walk-in coolers/freezers with 8’ ceilings must have semi-recessed sprinkler heads.

- Sprinkler Contractor must install wire guards over sprinkler heads on 8’-0” ceilings or lower throughout the entire store.

- The painting contractor must purchase all painting supplies thru the national account that Food Lion has with Sherwin Williams. The tracking parent code for Food Lion development and capital projects that are subcontracted is National Parent #C137. Open a job # on the painters account called “Food Lion”. Email the account and job # detail to james.cooley@sherwin.com. The job # will then be attached to parent #C137.

Front Exterior

- General Contractor must install and set the parking lot cart corrals a total of 6 spaces out, anchor down, and secure signage. These are to be set with one corral per parking bay.

- General Contractor is to have all storefront glass professionally cleaned after pressure washing the sidewalk during the week prior to Grand Opening.

- General Contractor is to steam clean front sidewalks, storefront masonry, vinyl ceilings, and dumpster pads one week prior to grand opening using hot water.

- General Contractor is to caulk at sidewalk and around columns at storefront with “NP” type compound.

- General Contractor is to steam clean front sidewalks, storefront masonry, vinyl ceilings, and dumpster pads at end of project (coordinate with Construction Manager), must use hot water.

- Electrical Contractor is to coordinate with the Building Sign Installer for power disconnection/connection. The quantity of circuits shall be per the electrical plans. Circuit junction box for reuse should be mounted above the canopy, on the exterior wall, approximately center of the sign location. MC cable whip to the sign company’s wiring trough will be completed by the sign company and verified by the Electrical Contractor. The new sign is 24vDC and all wiring to the individual letters is by the Sign Installer.

Vestibule And Sales Area

- General Contractor is to provide and install Acrovyn wall covering on all exposed walls and corners up to 4’ A.F.F.

- General Contractor is to fill in the voids on the top of the angled cases and shelf as needed.

- General Contractor will build drywall and metal stud chases around the refrigeration lines over the frozen food area as required.

- General Contractor will caulk around all doorframes.  This is after all the VCT work has been successfully completed.

- General Contractor needs to caulk around the top of all stainless steel column wraps in the sales area and the seams of the column wraps with clear silicone.  This needs to be done after all painting has been successfully completed on the sales area columns. The general contractor will sand and grind the sales area columns to remove runs in primer, existing brackets, conduit clamps, etc. General Contractor is to repair holes in columns before painting.

- General Contractor is to install plywood caps over all gaps between shelving the length of the shelving and in gaps between angled cases.

- Décor vendor will install track to the bottom of trellises.

- Plumbing Contractor is to run a 1/2“water line for the produce case quick connects for case washing capability.

- General Contractor should contact Portico directly to coordinate installation date of the Vestibule Carpet.  Contact is Greg Griffen - (877) 285-8899.  NOTE: LEAVE A 5% OVERAGE FOR FUTURE USE. The automatic doors must also be adjusted to allow for egress break away when installing the carpet tiles, if applicable.  Portico to provide new aluminum thresholds at all locations and anchor with flat head Stainless steel screws.

- Painting Contractor is to include painting all new refrigeration drip pans, electrical conduit, mechanical duct, security camera mounting and conduit, unistrut for décor and lighting, and any structural support added in the exposed structure ceiling area visible to customers.

- General Contractor to ensure the full perimeter light trough is sealed in the exposed structure area of the sales floor to separate the conditioned sales area from non-conditioned air spaces.

- Electrical Contractor to coordinate ordering, delivery, storage and installation of all lighting shown on the fixture plans. Lighting will be provided through Food Lion, by Cree Lighting.  Hang from structural steel and provide clean anodized electrical accessories.

- General Contractor is to include the material and installation of all Finish Metal work required.  Acceptable standards are:

  - 1/8” 3003 polished aluminum (PVC coated) on all exposed corners across the storefront including boxed column wraps.

  - 1/8” 3003 polished aluminum (PVC coated) thresholds at all Deli openings, Meat Prep to Sales Area opening, Produce Prep to Sales Area opening, and any Sales Area to backroom opening.

  - 1/8” 3003 polished aluminum (PVC coated) 4' corners on all outside Deli hard tile corners.

  - 1/8” 3003 polished aluminum (PVC coated) 4’ corner guards on all sales area exit door openings, Produce Prep door opening, and Meat Prep Door opening.

  - 1/8” 3003 polished aluminum (PVC coated) Restroom Corridor opening wrap that wraps 2” on the sales area side, through the opening and wraps 2” on the Corridor side to create a full cap.

  - 1/8” 3003 polished aluminum (PVC coated) 4’ corner guards on any Restroom Corridor exit door openings, restroom door openings, backroom traffic door opening, and any other Restroom Corridor door openings, include a 4’ corner on any outside wall corner in corridor.

  - Cooler/Freezer door(s) on the dairy cooler/freezer are to get diamond aluminum plate 4’ A.F.F.  (customer facing only).

  - Provide and install stainless steel kick plate the length of the rear load dairy coolers front and sides.

  - Cooler wall tread plate in the dairy cooler shall extend 12” up wall and turn 2” on floor, mechanically attach to floor.

- General Contractor to caulk around all doorframes.  This is after all the VCT work has been successfully completed.  The color of the caulk needs to be coordinated with the Construction Manager.

- Soft Tile Contractor is to include moisture test, grinding of any loose patch/high spots/problem areas, sanding entire floor, installation of Mapei Plani-Seal Easy, floor patch and leveler, and final sanding in the VCT bid, no change order will be accepted for floor patch.

Office Workroom / Data Closet

- Electrical Contractor will coordinate the installation and schedule of the fire and security conduit with the alarm vendor installers.

- General Contractor is to provide and install a door scope in the office door.

Data Cabinet

- Note:  The UPS system will be provided by Food Lion and will be connected by a Food Lion subcontractor.  Electrical Contractor will provide wiring as shown on the drawings.

- The installation of the data cabinet and support system will be performed by others. The electrician will have responsibility to extending all conduit and wiring to the cabinet per plans.

- The general contractor will properly store the UPS unit and switch once this is delivered until the Food Lion subcontractor arrives at the project to install.

Deli/Bakery Prep & Deli/Bakery Sales Area

- General Contractor to install aluminum corner guards on ALL exposed outside tile corners 4’ high.

- Electrical Contractor is to properly label both ends of ALL scale cabling with visible identification tags. Coordinate scale locations with drawings or with the Construction Manager ONLY!

- Plumbing Contractor must run individual condensate drain lines off evaporators. NO evaporate drain lines can be tee’d together.  ALL evaporator condensate lines are to be anchored to the wall and hanger supports from the ceiling. Install plumbing escutcheons on wall penetrations. Minimum slope for condensate lines are 1" per foot in coolers and 12" per foot in freezers.

Produce Prep & Produce Sales Area

- Electrical Contractor shall install an 110V duplex receptacle at 12” AFF under the Single-Deck Produce cases and an 110V WP GFI duplex receptacle at 60” AFF in the Produce Prep (Coordinate the exact location with the Food Lion Construction Manager).

- Plumbing Contractor shall install a ½” cold water line with a FPT ball valve at the Single-Deck Produce cases 8” AFF and 7’ AFF in the Produce Prep (Coordinate the exact location with the Food Lion Construction Manager). The misting system for the Single-Deck Produce cases and the humidifying system for the Produce Cooler are to be installed by others.

Meat Prep & Meat Sales area

- Plumbing Contractor must have all water lines, vent pipes and electrical conduit concealed in the wall; no exposed lines or conduit.

- Any surface mounted conduit must use stand-off brackets.

Restroom Corridor

- General Contractor is to install Acrovyn 4’ high in restroom corridor. Provide hat channel and ½” plywood, poly, and Acrovyn on exterior walls and ½” plywood and Acrovyn on the remaining interior walls. Provide wall mounted crash rail (black), aluminum corner guards and vinyl base. (NO surface mounted conduit will be accepted) All hat channel to be mounted horizontally.

Restrooms

- General Contractor is to install new baby changing station stickers on the restroom doors and new baby changer stations mounted top height of 48” AFF. Follow installation guide rules included with each station to ensure proper mounting height.

- Plumbing Contractor must order “bone” color toilets, urinals, and lavatories.

- General Contractor to order stainless steel hand dryers from George Scott & Associates.

- Plumbing Contractor is to install waste line padded covers under all sinks in both restrooms.

##### Grocery Storage Area

- General Contractor shall leave a set of as-built construction drawings in the motor room at the end of the project.  These are to be in a 6” PVC sleeve with threaded end caps and the final completion date written on the PVC sleeve mounted horizontally to the wall.

- General Contractor to assemble and install:  Recycle bins, cart corrals (6-spaces out, anchor and secure signage), benches, misc. tables, ALL trashcans, lounge furniture, managers office furniture, etc.  Coordinate with Construction Manager.

- General Contractor to seal the demising wall between the sales area and backroom areas. All dry wall penetrations should be sealed and filled. Also, any existing penetrations need to be sealed in the motor room.

- General Contractor is to remove/clean all unused conduit, wire, and construction debris from the bar joist in the truck dock, grocery storage, and mechanical rooms.

- Plumbing Contractor is to provide and install check valves on the cold water side of all mixing valves. (Include all prep sinks with hand sprayers)

- Tile/General Contractor to install 2” wide min. flat stock aluminum threshold at all VCT transitions.

- Painting Contractor must paint the grocery storage columns up to the bottom of the bar joist and paint traffic yellow 4’-0” AFF.

- General Contractor to construct a back-stock storage platform.  Platform to be constructed of 4x8x¾" plywood with 2x4 supports (turned tall) with six threaded rod passing through the plywood and 2x4 with washer and two bolts supported by unistrut attached to the roof joist.  Coordinate location with Construction Manager, Do Not Place Above Cooler/Freezers.

##### Lounge

- General Contractor will install Acrovyn in the lounge using appropriate Acrovyn wall/trim.

- General Contractor is to provide and install corner cabinet in the lounge. Electrical Contractor is to provide (2) dirty power receptacles above cabinet and to be flush mounted in the wall.

- General Contractor is to provide 10 sturdy coat hooks evenly spaced on a (clear coated pine) board.

- General Contractor to install two cork boards provided by Food Lion.

- General Contractor is to install a paper towel dispenser in the lounge. Coordinate the location with the Construction Manager. (Food Lion will provide the dispenser)

##### Department Managers Office

- General Contractor is to provide and install Acrovyn full height wall/trim over hat channel ply-wood/sheetrock.

- General Contractor to install push/pull plate hardware on door.

- Electrical Contractor is to provide and install wall box and conduit with pull string for Muzak to install mic above counter top.

Roof

- General Contractor to seal any abandoned penetrations on roof. All work must comply with roof warranty.

- General Contractor to install rubber boots on all roof penetrations and galvanized sheet metal over all existing roof curbs.

- General Contractor to remove all debris from the roof one week prior to store opening.

- Painting Contractor to scrape, prime and paint rooftop steel.

Rear Exterior & Sides

- General Contractor is to locate down spouts at new delivery ramps so they are relocated at least 5’ past the end of the ramp and do not empty on or near the end of the ramp.  Minimum downspout clearance above ramp shall be 8’.

- General Contractor is to properly re-stripe truck guide stripes (as per Food Lion Plans and Specifications).  Provide glass beads for stripping.

- General Contractor is to patch any voids in the block prior to painting the exterior walls.

- General Contractor is to remove the existing handrails and replace with new handrails at the DSD door and ramp.

- General Contractor is to install concrete splash guards around the rear of the store over the asphalt.  Splashguards are to be silicone to the asphalt and silicone around edges to hold its position.

- General Contractor is to stencil the store numbers on the back dock doors including the new DSD door and roll-up doors.

- General Contractor is to caulk the joints at the truck pad and the rear walls.

- General Contractor/Painting Contractor is to paint the painted CMU walls Paint color to match existing.

- General Contractor is to install fly fans over DSD and driver’s door, wired by electrician.

Mechanical Room

- General Contractor shall leave a set of as-built construction drawings in the mechanical room at the end of the project.  These are to be in a 6” PVC sleeve with threaded end caps and the final completion date written on the PVC sleeve mounted horizontally on the wall.

- General Contractor to install new weather stripping and sweeps on the motor room doors.

Project Specific Notes

- Clean lens of front soffit lighting.

- Pressure wash the vinyl soffit.

- Undercut the interior vestibule doors for carpet clearance.

- Reuse and repaint the vestibule grid and install new ceiling tile and light fixtures.

- Paint perimeter walls full height.

- The entire deli space and drop ceiling will remain in place.

- Remove all existing VCT in the sales area.

- Furnish and install new sales area stainless steel column wraps. Caulk top and bottom of these.

- Remove floor slab and install new frozen food case infrastructure.

- Replace all existing impact doors throughout the store. The new impact doors will be provided by Mueller through Food Lion’s national account program. Contractor to price demo and new install.

- The general contractor will be responsible for the removal and disposition of the existing storage freezer.

- New dock seals to be furnished through Food Lion’s national account program.

- Remove and replace the existing dock doors. The new Overhead doors will be provided through Food Lion’s national account program.

- New fly fans will be installed at the roll-up dock doors and wired appropriately.

- The existing dock levelers will be reused.

- Paint all exterior metal.

- Approximately 14 telepoles will be needed for the front-end checkstands, deli island cases, produce island cases and the fresh pet case.

- Install new handicap parking signs on the existing Unistrut in the parking lot.

- Change out the deli HVAC diffusers per the plans.

- Remove the existing seafood trellis.

- Add receptacles at the remaining island promos in the rear isle as per the plans.

- The existing meat prep light fixtures will remain. The electrical contractor will relamp these fixtures and clean the lenses.

- Remove the existing chain link fence near the produce prep.

- Install 2x6 wood base in the produce prep room.

- Clean all FRP throughout the store and caulk holes as needed.

- The existing gas dock heaters are to remain in the backroom.

- Clean and point up the existing drywall in the existing upstairs office and repaint entire room.

- Remove the platform at the trash compactor and infill the opening with masonry.

- Remove the expectant mother parking sign and install an additional handicap sign.

- Cut all utility drops up to the bottom of the joists throughout the sales area.

- Adjust the existing meat case drains and produce case drains as needed.

- Wrap the utility drops at the rear island cases with PVC enclosures.

- Repaint the associate restroom in the grocery storage area.

- Paint new FDA stripe throughout the backroom. Do not repaint the backroom masonry.

- The general contractor is responsible for removing and disposal of all remaining millwork, checkout stands, deli seating, etc.

- Paint the new refrigeration drain pans regardless of whether the dryfall painting alternate is accepted or not.

- Cap the drains at the existing grinder and prep sink in the meat prep room.

- Relocate the fire suppression heads under the deli exhaust hood as needed.

--------------------------------------------------------------------------------------------------------------------

###### Allowances

---------------------------------------------------------------------------------------------------------------------

*Note: General Contractor is to incorporate in this contract the listed allowance as stated in the bid notes per the Food Lion Construction Manager.

---------------------------------------------------------------------------------------------------------------------

- Add an $8,000 allowance for repairs, and rework of existing deli and meat prep epoxy flooring.

- An allowance of $3,000 should be included for new stainless steel corners, thresholds, etc. in the deli.

--------------------------------------------------------------------------------------------------------------------

###### Alternates

---------------------------------------------------------------------------------------------------------------------

- Include an alternate for the installation of new sheetrock, vapor barrier, and hat-channel on the exposed block walls in the sales area. This should also include finishing and painting drywall.

- As an alternate, include pricing for dryfall painting of roof structure, decking, ductwork, conduit, sprinkler piping, etc. overhead in the sales area.

- Price the renovation of the front restrooms as an alternate including fixtures, finishes, partitions, etc.

- All of the above alternates should include general conditions, insurance, overhead and profit for each specific item.

CC:	Construction Manager – Clarence Ballard

General Contractor – Ashland, Salcoa, Vertex, Wimco & Windle

Maintenance Manager – Steve Millard

Maintenance Supervisor – Jim Carter

2250walk1

CB/slh